EEP Documentation of Data Compilation Workflow

Reservoirs

- Data Source: USACE National Dam Inventory (2013)

  - Create an account to get access

  - Download federal, local, state, utility, private, and not listed reservoirs

  - Extract reservoirs located in North Carolina, Virginia, Tennessee, and South Carolina (shared basins)

    - Remove duplicate NIDID numbers. Make sure storage is not deleted in the process. Some duplicate NIDID numbers don’t duplicate other records.

      - Sort by NIDID number and storage volume. Create a flag column and flag those with duplicates. Remove. Duplicates are often levees around the dams or saddle dikes.

    - Remove those missing lat/longs – tend to be ponds (ash ponds, fossil fuel complex, etc)

    - Clip to study area

Number of Reservoirs by State

| Type | Phase | NC | SC | TN | VA | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Federal | Filter by state | 63 | 41 | 96 | 68 | 268 |
|  | Remove Duplicates | 56 | 40 | 54 | 64 | 214 |
|  | Clip to study area | 56 | 40 | 24 | 19 | 139 |
| State | Filter by state | 61 | 78 | 155 | 74 | 368 |
|  | Remove Duplicates | 61 | 77 | 148 | 74 | 360 |
|  | Clip to study area | 61 | 77 | 5 | 18 | 161 |
| Local | Filter by state | 286 | 182 | 177 | 271 | 916 |
|  | Remove Duplicates | 286 | 179 | 174 | 265 | 904 |
|  | Clip to study area | 286 | 179 | 15 | 73 | 553 |
| Utility | Filter by state | 101 | 46 | 9 | 52 | 208 |
|  | Remove Duplicates | 86 | 25 | 9 | 50 | 170 |
|  | Clip to study area | 86 | 24 | 2 | 17 | 129 |
| Private | Filter by state | 2610 | 2089 | 793 | 1184 | 6676 |
|  | Remove Duplicates | 2603 | 2062 | 789 | 1166 | 6621 |
|  | Clip to study area | 2603 | 2058 | 181 | 302 | 5144 |
| Not Listed | Filter by state | 141 | 0 | 0 | 413 | 554 |
|  | Remove Duplicates | 141 | 0 | 0 | 402 | 543 |
|  | Clip to study area | 141 | 0 | 0 | 46 | 187 |

Total Number of Reservoirs per state:

NC – 3,233 dams

SC – 2,378 dams

TN – 227 dams

VA – 475 dams

TOTAL:  6,313 dams

- Estimate which reservoirs are removable

  - Assume not removable: Size > 500 ACFT

    - Possible removeable Size: <500 ACFT

    - Assume easily removable size: <100 ACFT

57% of reservoirs are smaller than 100 ac-ft

88% of reservoirs are smaller than 500 ac-ft

Create estimates for the reservoirs without storage estimates (50, 120, and 550 – puts it into categories of removability)

Base on Owner Type and NID height. Cells highlighted in red.

Rare Species

- Obtain from NCOneMaps: nheo and nhpna

- NHEO – National Heritage Element Occurrence Data

  - Accuracy: percentage of the polygon that is occupied by the species

  - EO_Status: Current, Historical, Obscure

  - EO_Rank: Estimates the viability of the element occurrence

    - A: Excellent, B: Good, C: Fair, D: Poor

    - E: Exists but insufficient information to rank

    - F: Failed to relocate EO but may still exist

    - H: EO is old, failed to find it in a recent survey but may still exist

    - NR: Not rated, U: Not rated, ?: Not rated

    - X: EO has been destroyed

    - i: EO was introduced – used as a qualifier with above

    - r: EO was reintroduced or restored – used as a qualifier with above

  - Last observed date

  - Name_Catgy: categorizes into taxonomic groups

NHEO processing

- Name Category:

  - Animal + Animal Assemblage (n=7,391)

  - Natural Community (n=7,034)

  - Plant (n=10,649)

- Processing of EO_Status

  - Historical (n=3,692)

    - EO_Rank (letters with question marks – removed ? since best guess)

      - F: 474, Fi: 3, Fr: 4

      - H: 2,089, Hi: 3, Hr: 1

      - X: 390, Xi: 5, Xr: 3

  - Obscure (413)

    - All NR and U – some have very old dates and some younger

  - Current (20,969) – some current’s go back to 1905

    - A: 2,950, Ai: 1, Ar: 2

    - AB: 732, ABi: 2, ABr: 2

    - AC: 40

    - B: 3,543, Bi: 9, Br: 14

    - BC: 1,262, BCi: 1, BCr: 2

    - BD: 40

    - C: 3,619, Ci: 1, Cr: 7

    - CD: 751, CDi: 1, CDr: 1

    - D: 2,121, Di: 23, Dr: 8

    - E: 5,803, Ei: 12, Er: 4

Processing Thoughts:

- Delete obscure sites

- Rank Historical sites low (+0.5)

- Rank Current sites higher

  - Healthy sites are A & B sites – so would these be conservation oriented? (+1)

    - Merge “i” and “r” status into the letter b/c so few

  - Less healthy sites are C, D, F – so would these be restoration priorities (+2)

  - Unknown sites – how to rank those?

- Accuracy – do we want to multiply the factor by the accuracy level? Weight it that way?

- Processing of Name Category

  - If the site is a plant – only will benefit from riparian corridor or floodplain connection management handles.

  - Animals or natural communities may benefit from all

nhpna: Natural Heritage Program Natural Areas

Attributes:

- R_Rating – Element Representational Rating (a natural area’s potential to contribute to a collection of the best locations of reach tracked element within the state)

  - R1 – exceptional

  - R2 – very high

  - R3 – High

  - R4 – Moderate

  - R5 – General

  - R? – Unknown

- C_Rating – Element Collective rating – evaluates the conservation value of each natural area based on the number of tracked elements present, rarity of elements, weight in terms of both global imperilment and state imperilment.

  - C1 - Exceptional – 10 or more elements

  - C2 – Very high – 7 or more elements

  - C3 – High – 4 or more elements

  - C4 – Moderate – 2 or more elements

  - C5 – General – 1 or more elements

  - C? – Unknown

- Owner – includes land conservancies, colleges, conservation trusts, indian reservations, State, etc

- Top_Rating – Highest of the two element ratings

Processing Thought

Use the Top_Rating as a weighting mechanism

Duke University List of T&E, Candidates and Petitioned Species by Taxon

- The spreadsheet lists species by county FIPS code and HUC8

  - In GIS  Union together HUC8 boundaries and County Boundaries

  - GIS FIPS codes does not include the state id (37)

- In the spreadsheet – create a NC FIPS code by removing the 37 in order to later join to the Union Shapefile.

  - In the spreadsheet concatenate together HUC8_NCFIPS

    - Assign NA values to any fields that only have a HUC8 or only have a FIPS code (not all entries have both a FIPS and a HUC8.

  - In the GIS create field that concatenates together the HUC8 and the FIPS code

- In R  Overlays.R run the scrip that:

  - Separates out species by their ESA status

    - E: Endangered

    - T: Threatened

    - SAT: Threatened Due to Similarity of Appearance – not subject to section 7

    - Candidate: Candidate species for official listing

    - Petitioned: Species petitioned for official listing

  - Create summary tables by fips, huc8, and HUC8_FIPS categories (no duplication)

    - Table lists the identifier and the number of listings for a taxon (12 taxons given).

    - (There are 5 ESA status and 3 locational identifiers = 15 csv files outputted)

- Not sure best approach

  - Summarize by Taxon Group for Management Handle Purpose

    - aquatic (amphibian, crustacean, fish, mussel)

    - upland (Arachnid, Bird, Insect, Mammal, Non-Vascular Plant, Reptile, Snail, Vascular Plant)

  - Then Summarize those groups by status

    - E&T

    - SAT, Can, Pet

  - Link layers to the GIS file

    - Match by HUC8_FIPs

    - Not sure how to easily group the others

      - FIPS – link to anything with that FIPS

      - HUC8 – link to anything with that HUC8

    - Then add those three up within the spatial layer

  - How to weight once added together?

Nhpna: blue;       nheo: current (green), historical (pink), obscure (white)

--- I don’t like that some are just counties for the nheo.

Important Coastal Conservation Areas

Coastal Habitat Protection Plan

- If search in NCOnemap – points to Strategic Habitat Areas

- Submerged Aquatic vegetation is also listed as being important – it is only located at the very edge of the coastal plain and is sensitive to water quality, etc that is determined upstream.

  - I don’t think this layer will be helpful.

- Other data we could include:

  - Shellfish growing areas

  - Estuarine Benthic Habitat mapping (gives the strata – don’t know how to link to benthos health)

Division of Marine Fisheries Strategic Habitat Areas

- Download both shapefiles from NCOnemaps

  - No strategic habitat areas defined for the lumber or cape fear basins

- No attributes to use – just presence / absence

Important Mountain Conservation Areas

- Public Mountain Trout Waters downloaded from NCOnemaps

  - Location (streams/lakes) of waterbodies containing trout, water class, and associated regulations

Processing: Both of these are weighted by presence / absence

Important Conservation Areas: General

High Quality Waters and Outstanding Resource Water Management Zones (hqworw)

- Can Weight by Class (in order of quality)

  - ORW + ORW special management

  - HWQ – High Quality Waters

  - SA & WSW – includes water supply watersheds (WS1 or WSII) and shellfishing areas

- A polygon can have more than one class

Wildlife Resources Commission Priority Areas

- Could not find any data for it – need to ask EEP

Land managed for conservation and open space

- This seems to overlap with the conservation land set aside for likelihood for project to succeed

Significant Areas for Ecosystem SErvices

- Potential EnviroAtlas Layers (hunting layers):

  - Mean small game species richness

  - Mean total harvestable species richness

  - NIB big game species richness

- Hiking / Vista layers: maybe location of parks (covered in marea / conservation shapefile)

Likelihood for Project

Current Likelihood

- Land Valuation

  - Download data from quickstats.nass.usda.gov – rent in units of $/acre

    - Irrigated land, non-irrigated land, and pastureland

    - Obtained data from 2008 – present

Clean Water Management Trust Fund Projects (CTPC)

- Download from nconemaps (Conservation Tax Credit Properties)

- Names Recipient name of the land, donor type, and the Conservation Details

  - E.g. Forest Preservation, Agricultural Preservation / Water Quality…

Land Trust Conservation Property (LTCP) – in marea file already

- Download from NCOnemaps (Land Trust Conservation Properties)

- Names Land Trust that owns the property

Managed Areas in North Carolina (MAREA)

- Download from NCOnemaps (Managed Areas in North Carolina)

- Owner Type, Category (Fee, Easement, Other), GAP_STATUS (measure of intent to conserve biodiversity with 1 being very high and 4 being no institutional mandates or legally recognized restrictions)

Marea Includes LTCP but not CTCP

Marea includes EEPEasements, IMS_ID_Conservation, SPO_ID_Conservation

but not Conservation_Easement_Pin (point not a polygon)

EEP Projects

- Conservation_Easement_Pin

  - Can do by sub-type: Complete-Build Ready, In Progress, Control Point

    - Not in same location as EEPEasements shapefile

- Project Site Location (point file) and status of the project

- Areas associated with the project (EEPProjectArea):

  - Type: BMP, Channel Feature, Constraint, or Vegetation

- Buffer Area (EEPBufferArea):

  - Buffer Type: Encroachment, Existing Vegetation, Planted Trees, Replanting Zone

UNABLE TO FIND:

Downloadable 319 project sites (DENR has a map of locations)

Mitigation Bank sites

Agricultural BMP’s – can get a few off the EEP data we have, but not many

Threats / Future Development

- Projected Population Change

  - EPA’s ICLUS dataset (population projections for every 5 years)

    - http://cfpub.epa.gov/ncea/global/recordisplay.cfm?deid=257306

    - County population projects

    - Housing Density Projections (grid)

      - GIS tool that lets you create population projection rasters from their data

- DOT Transportation Improvement Program

  - http://www.ncdot.gov/bikeped/funding/process/

DOT has a spatial data file of upcoming road and bridge improvements but don’t have access to it online. The PDF has estimated timing of projects.